Drone Airspace Map: Data Sources

Every dataset the tool uses, every FAA dataset it does not use, and what was pulled from each, with FAA field names explained.

Tools covered: build_uas_layers.py (downloads and normalizes data into out/uas_rules.gpkg), make_uas_map.py (HTML map) and uas_app.py (Streamlit map). Data as of 1 October 2026. New York counts are for the bounding box -79.8, 40.4, -71.8, 45.1.

How to read the field tables: "Field" is the exact name in the FAA data. "Example value" is a real record pulled on 1 October 2026. "What it means" explains the field; where the FAA does not document a field, the meaning is inferred from its name and values and is marked as such. "How the tool uses it" names the normalized column it feeds (see section 1). Contact fields (POC) are left out of the examples.

# 1. Summary

|  | Count | Notes |
| --- | --- | --- |
| Datasets used | 15 + 1 | 10 FAA ArcGIS layers for rules, 1 FAA ArcGIS layer for airports, SEAMS, and 3 tfr.faa.gov feeds; plus the FAA NOTAM API (set up, not yet pulled) |
| FAA ArcGIS services not used | 95 | Of 106 services on the FAA UAS/aeronautical ArcGIS server (services6.arcgis.com/ssFJjBXIUyZDrSYZ), 11 are used |
| Non-FAA services | 3 | Esri basemap tiles, Open-Meteo weather, Leaflet map library |

## Common columns the tool adds

Every downloaded feature is rewritten into the same columns so layers can be stacked. The original FAA fields are kept unchanged in source_attrs.

| Column | Meaning |
| --- | --- |
| layer, source_id, name | Which layer, the FAA record ID, and a readable name |
| effect | prohibited · restricted · authorization_required · caution · allowed (assigned by the tool from the FAA fields, as described for each dataset) |
| floor_ft, floor_ref, ceiling_ft, ceiling_ref | Vertical limits in feet, with reference AGL (above ground), MSL (above sea level), FL (flight level) or UNL (unlimited) |
| window_start, window_end | When the rule applies (UTC). Empty = permanent |
| notes, citation | Plain-language notes; the regulation the tool cites (best effort, should be checked) |
| confidence | high = taken directly from FAA fields; medium = derived (e.g. stadium circle, multi-area TFR); low = parsed or approximated |
| source_url, source_last_edit, fetched_utc | Where it came from, the server's own last-edit date, and when it was downloaded |
| source_attrs | All original FAA fields, as JSON |

Server metadata read for every ArcGIS layer: editingInfo.dataLastEditDate (falls back to editingInfo.lastEditDate) → source_last_edit and the manifest's server_last_edit; maxRecordCount → download page size.

# 2. Datasets used

## 1. UAS Facility Maps (LAANC grid)

Service: FAA_UAS_FacilityMap_Data

Layer: FAA_UAS_FacilityMap_Data (layer 0)

Source: FAA UAS Data Delivery System (ArcGIS FeatureServer, services6.arcgis.com/ssFJjBXIUyZDrSYZ)

Refresh group: weekly

Output layer: laanc_grid

Records: 380,860 grid cells (U.S.); New York area: 21,932 cells (merged into 266 shapes for the map)

What it is: A grid of roughly 1-by-1 arc-minute squares around controlled airports. Each square carries the maximum altitude the FAA will pre-approve for drone flights there through LAANC (Low Altitude Authorization and Notification Capability).

What was pulled: Only these fields were requested (outFields): OBJECTID, GLOBALID, CEILING, UNIT, MAP_EFF, LAST_EDIT, APT1_FAAID, APT1_NAME, APT1_LAANC, AIRSPACE_1, REGION, plus the polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| CEILING | 400 | Maximum altitude (in UNIT, above ground level) that can be authorized automatically in this square. 0 = no automatic approval; a manual FAADroneZone request is needed. | ceiling_ft (AGL); colors the grid |
| UNIT | Feet | Unit of CEILING. | source_attrs |
| APT1_FAAID | ELM | FAA location identifier of the primary airport the square belongs to. | Part of name; LAANC shapes merged by this + CEILING |
| APT1_NAME | Elmira/Corning Rgnl | Name of that airport. | name |
| APT1_LAANC | 1 | Flag for LAANC availability at that airport (1 = yes; inferred from the field name and values). | source_attrs |
| AIRSPACE_1 | D | Class of the controlled airspace the square sits in (B, C, D or E). | notes |
| MAP_EFF | 9/3/2026 | Effective date of the current facility map. | source_attrs |
| LAST_EDIT | 1/4/2018 | Date this square was last edited. | source_attrs |
| REGION | Eastern | FAA region responsible for the map. | source_attrs |
| OBJECTID, GLOBALID | 06bc2626-… | Internal and global unique IDs. | source_id |

Not pulled / not used: Not requested: APT2–APT5 (_FAAID, _ICAO, _NAME, _LAANC, _Enabled: secondary airports sharing the square), ARPT_COUNT, AIRS_COUNT, AIRSPACE_2–5, APT1_ICAO, LATITUDE/LONGITUDE (square center), Shape fields.

Rule assigned by the tool: authorization_required, citation 14 CFR 107.41.

## 2. Class Airspace

Service: Class_Airspace

Layer: Class_Airspace (layer 0)

Source: FAA UAS Data Delivery System / Aeronautical Data (same ArcGIS server)

Refresh group: weekly

Output layer: class_airspace

Records: 6,045 polygons; New York area: 370 polygons

What it is: Boundaries of Class B, C, D and E airspace as drawn on FAA aeronautical charts, with their floors and ceilings.

What was pulled: All fields (outFields = *) and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NAME | NEW YORK CLASS B | Name of the airspace. | name |
| CLASS | B | Airspace class letter (A, B, C, D, E, G). | Decides effect |
| LOCAL_TYPE | CLASS_B | FAA sub-type. CLASS_E2 = Class E surface area; CLASS_E3/E4 = surface extensions; CLASS_E5 = Class E starting at 700 or 1,200 ft; CLASS_E6 = en-route domestic; MODE C = Mode C veil. | Decides effect |
| TYPE_CODE | CLASS | Feature type: CLASS (class airspace), CTR, TMA, MODE-C, etc. | Decides effect |
| IDENT / ICAO_ID | JFK / KJFK | Identifier of the associated airport (FAA and ICAO codes). | source_attrs |
| UPPER_VAL / LOWER_VAL | 7000 / 0 | Numeric top and bottom of the airspace. | ceiling_ft / floor_ft |
| UPPER_UOM / LOWER_UOM | FT | Unit of the value: FT (feet) or FL (flight level, hundreds of feet). | FL values are multiplied by 100 |
| UPPER_CODE / LOWER_CODE | MSL / SFC | Reference for the value: SFC (surface, i.e. above ground), MSL (above mean sea level), STD (standard pressure = flight level), UNLTD (no upper limit). | ceiling_ref / floor_ref (AGL, MSL, FL, UNL) |
| UPPER_DESC / LOWER_DESC | TI | How the limit is applied: TI = "to and including", TNI = "to but not including", ANI = "above but not including" (codes as they appear in the data; FAA does not publish a legend in the service). | Kept in source_attrs |
| SECTOR | AREA A | Named sub-area (Class B shelves are split into lettered areas). | source_attrs |
| WKHR_CODE / WKHR_RMK | H24 / — | Hours of operation. H24 = continuous; RMK = see the remark text (e.g. part-time Class D). | notes |
| MIL_CODE | CIV | Civil, military or joint use. | source_attrs |
| COMM_NAME / CONT_AGENT | — | Communications facility / controlling agency, when given. | source_attrs |
| CITY, STATE, COUNTRY | NEW YORK, NY | Location. | source_attrs |
| ADHP_ID | 6AB3CFCC-… | Link to the airport record (ADHP = aerodrome/heliport). | source_attrs |
| LEVEL_ / LEVEL_CODE | L | Which altitude structure it belongs to: L = low, H = high, B = both. | source_attrs |
| ONSHORE | 1 | 1 = over land, 0 = offshore. | source_attrs |
| EXCLUSION | 0 | 1 = this polygon is a cut-out (an area excluded from the surrounding airspace). | source_attrs |
| US_LOW, US_HIGH, AK_LOW, AK_HIGH, US_AREA, PACIFIC | US_LOW = 1 | Flags (1/0) for which FAA chart series shows the feature (U.S. low/high en-route, Alaska low/high, area charts, Pacific). | source_attrs |
| GMTOFFSET / DST / DST_CODE | -5 / 1 | Local time-zone offset from UTC and whether daylight saving time applies; used to interpret local times of use. | source_attrs |
| Shape__Area, Shape__Length | 0.0183 | Polygon area and perimeter in the service's units (degrees). Technical, not meaningful to users. | Dropped in the map |

Rule assigned by the tool: authorization_required for Class B, C, D and E2–E4 (surface-based); caution for everything else (Class A, E5, E6, Mode C). Citation 14 CFR 107.41; 14 CFR Part 71.

## 3. Special Use Airspace

Service: Special_Use_Airspace

Layer: Special_Use_Airspace (layer 0)

Source: FAA ArcGIS server

Refresh group: weekly

Output layer: special_use_airspace

Records: 1,542 polygons; New York area: 35 polygons

What it is: Military and other special-purpose airspace: restricted areas, military operations areas (MOAs), warning, alert, danger and prohibited areas.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NAME | R-5201 | Designator of the area. | name |
| TYPE_CODE | R | Type of special use airspace (see codes below). | effect |
| UPPER_VAL / LOWER_VAL | 7000 / 0 | Numeric top and bottom of the airspace. | ceiling_ft / floor_ft |
| UPPER_UOM / LOWER_UOM | FT | Unit of the value: FT (feet) or FL (flight level, hundreds of feet). | FL values are multiplied by 100 |
| UPPER_CODE / LOWER_CODE | MSL / SFC | Reference for the value: SFC (surface, i.e. above ground), MSL (above mean sea level), STD (standard pressure = flight level), UNLTD (no upper limit). | ceiling_ref / floor_ref (AGL, MSL, FL, UNL) |
| UPPER_DESC / LOWER_DESC | TI | How the limit is applied: TI = "to and including", TNI = "to but not including", ANI = "above but not including" (codes as they appear in the data; FAA does not publish a legend in the service). | Kept in source_attrs |
| TIMESOFUSE | CONTINUOUS | When the area is active (e.g. CONTINUOUS, INTERMITTENT BY NOTAM, specific hours). | notes |
| CONT_AGENT | FAA, BOSTON ARTCC | Controlling agency to contact for permission or activation status. | notes |
| REMARKS | — | Free-text remarks. | notes |
| CITY, STATE, COUNTRY | FORT DRUM, NY | Location. | source_attrs |
| LEVEL_ / LEVEL_CODE | L | Which altitude structure it belongs to: L = low, H = high, B = both. | source_attrs |
| ONSHORE | 1 | 1 = over land, 0 = offshore. | source_attrs |
| EXCLUSION | 0 | 1 = this polygon is a cut-out (an area excluded from the surrounding airspace). | source_attrs |
| US_LOW, US_HIGH, AK_LOW, AK_HIGH, US_AREA, PACIFIC | US_LOW = 1 | Flags (1/0) for which FAA chart series shows the feature (U.S. low/high en-route, Alaska low/high, area charts, Pacific). | source_attrs |
| GMTOFFSET / DST / DST_CODE | -5 / 1 | Local time-zone offset from UTC and whether daylight saving time applies; used to interpret local times of use. | source_attrs |
| Shape__Area, Shape__Length | 0.0183 | Polygon area and perimeter in the service's units (degrees). Technical, not meaningful to users. | Dropped in the map |

| Code | Meaning |
| --- | --- |
| P | Prohibited area: no flight at all → prohibited |
| R | Restricted area: hazardous activity; permission needed when active → restricted |
| MOA | Military operations area → caution |
| W | Warning area (usually offshore) → caution |
| A | Alert area → caution |
| D | Danger area → caution |

Rule assigned by the tool: From TYPE_CODE as above. Citations 14 CFR 73.83 (P), 14 CFR 73.13 (R), FAA Order JO 7400.10 (others).

## 4. Prohibited Areas

Service: Prohibited_Areas

Layer: Prohibited_Areas (layer 0)

Source: FAA ArcGIS server

Refresh group: weekly

Output layer: prohibited_areas

Records: 13 polygons; New York area: 0 (none in New York; e.g. P-56 is in DC)

What it is: The prohibited areas only (P-40, P-56, P-49, etc.). Same schema as Special Use Airspace.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NAME | P-56 | Designator. | name |
| TYPE_CODE | P | Always P (prohibited). | effect = prohibited |
| UPPER_VAL / UPPER_CODE | 18000 / MSL | Top of the prohibited area. | ceiling_ft / ceiling_ref |
| TIMESOFUSE | CONTINUOUS | When in force. | notes |
| (other fields) |  | Same as Special Use Airspace above. | source_attrs |

Rule assigned by the tool: prohibited; citation 14 CFR 73.83; 14 CFR 99.7.

## 5. National Security UAS Flight Restrictions

Service: DoD_Mar_13

Layer: National_Security_UAS_Flight_Restrictions (layer 0)

Source: FAA ArcGIS server (the FAA's published item for this dataset points to the service named DoD_Mar_13)

Refresh group: daily

Output layer: nsufr

Records: 2,282 polygons; New York area: 65 polygons

What it is: Permanent drone no-fly areas over military bases and other sensitive federal facilities, issued under 14 CFR 99.7 special security instructions.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| Proponent | DOD | Agency that requested the restriction (e.g. DOD, DOE, DOJ, DHS). | source_attrs |
| Branch | USA | Service or component (USA = U.S. Army, USAF, USN, USMC, …). | notes |
| Base | Fort Indiantown Gap | Installation name. | notes |
| Facility | Muir AAF | Specific facility covered. | name |
| Airspace | Class D | Class of the underlying airspace. | source_attrs |
| Reason | National Security | Stated basis. | notes |
| Floor / Ceiling | Surface / 400' AGL | Vertical limits as text. | Parsed to floor_ft / ceiling_ft + AGL/MSL |
| FAA_ID | 20161222-DOD-Fort Indiantown Gap-Muir AAF | FAA identifier; begins with a date (YYYYMMDD), apparently the date it was established. | source_id |
| State, County | PA, Lebanon | Location. | source_attrs |
| POC | (omitted here) | Point of contact for the facility; can contain names and phone numbers. | source_attrs |

Rule assigned by the tool: prohibited, always in effect; citation 14 CFR 99.7.

## 6. Part-Time National Security UAS Flight Restrictions

Service: Part_Time_National_Security_UAS_Flight_Restrictions

Layer: Part_Time_National_Security_UAS_Flight_Restrictions (layer 0)

Source: FAA ArcGIS server

Refresh group: daily

Output layer: nsufr_part_time

Records: 5 polygons; New York area: 0

What it is: Security restrictions that are only in force when activated (e.g. Military Ocean Terminal Concord and Sunny Point).

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| (same as #5) |  | Proponent, Branch, Base, Facility, Airspace, Reason, State, FAA_ID, POC, Floor, Ceiling, County. | as #5 |
| ALERTYPE | Intermittent | How the restriction is activated. | notes |
| ALERTTIME | 2026-09-06 07:00 UTC | When the activation alert was issued (epoch milliseconds in the data). | source_attrs |
| ACTIVETIME / ENDTIME | 2026-09-07 → 2026-09-12 | Start and end of the active period. | window_start / window_end |
| ADVISEINST / ADVISEAUTH / ADVISENOTE | (blank) | Advisory instructions, issuing authority and notes for the activation. | notes |

Rule assigned by the tool: prohibited during the active window; confidence medium when no window is published.

## 7. National Defense Airspace TFR Areas

Service: National_Defense_Airspace_TFR_Areas

Layer: National_Defense_Airspace_TFR_Areas (layer 0)

Source: FAA ArcGIS server

Refresh group: daily

Output layer: nda_tfr

Records: 8 polygons; New York area: 0

What it is: Pre-defined National Defense Airspace areas that become temporary flight restrictions when activated by NOTAM.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NAME | SAN ANGELO NATIONAL DEFENSE AIRSPACE TFR | Area name. | name |
| TYPE_CODE | DEF | Defense airspace. | source_attrs |
| LOCAL_TYPE | NDA_TFR | National Defense Airspace TFR. | source_attrs |
| WKHR_CODE / WKHR_RMK | RMK / BY NOTAM | Active only when a NOTAM activates it. | notes |
| CITY, STATE, COUNTRY | SAN ANGELO, TX | Location. | source_attrs |

Rule assigned by the tool: prohibited (when active); no altitudes in the data, so confidence medium; citation 14 CFR 99.7; 49 U.S.C. 40103(b)(3).

## 8. Stadiums

Service: Stadiums

Layer: Stadiums (layer 0)

Source: FAA ArcGIS server

Refresh group: weekly

Output layer: stadiums_3nm

Records: 204 points; New York area: 12 points

What it is: Locations of venues covered by the standing stadium TFR (MLB, NFL, NCAA Division I football, major motorsports).

What was pulled: All fields and point geometry. The tool draws a 3 NM circle around each point.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NAME | MetLife Stadium | Venue name. | name |
| LATITUDE / LONGITUDE | 40-48-48.73N / 074-04-27.81W | Location in degrees-minutes-seconds text (the point geometry is used instead). | source_attrs |
| CITY, STATE | East Rutherford, NJ | Location. | notes |
| STATUS_CODE | Open | Whether the venue is open. | notes |
| OPENING_ON | null | Opening date for venues not yet open. | source_attrs |

Rule assigned by the tool: prohibited only during qualifying events (3 NM, surface to 3,000 ft AGL, 1 hour before to 1 hour after); confidence medium because the circle is derived.

## 9. FAA-Recognized Identification Areas (FRIA)

Service: FAA_Recognized_Identification_Areas

Layer: FAA_Recognized_Identification_Areas_polygons (layer 0)

Source: FAA ArcGIS server

Refresh group: weekly

Output layer: fria

Records: 2,723 polygons; New York area: 296 polygons

What it is: Sites, mostly model-aircraft club fields, where drones without Remote ID may fly within visual line of sight (14 CFR Part 89).

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| refNumber | 2023-CBO-01561-FRIA | FAA approval number (CBO = community-based organization). | source_id |
| title | Swetland RC Flyers #4668 - Swetland Ln | Site name. | name |
| orgName | Academy of Model Aeronautics | Sponsoring organization. | notes |
| address1, address2, city, state, zipcode | Swetland Ln, Wyoming, PA | Site address. | notes / source_attrs |
| startDate / endDate | 2023-08-23 → 2027-08-23 | Period the FRIA approval is valid (four years in this example). | window_start / window_end |

Rule assigned by the tool: allowed; citation 14 CFR 89.115(b); 14 CFR Part 89 Subpart C.

## 10. Recreational Flyer Fixed Sites

Service: Recreational_Flyer_Fixed_Sites

Layer: Recreational_Flyer_Fixed_Sites (layer 0)

Source: FAA ArcGIS server

Refresh group: weekly

Output layer: recreational_fixed_sites

Records: 2,172 polygons; New York area: 265 polygons

What it is: Flying sites in controlled airspace where recreational flyers may operate under an agreement with the FAA / airport.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| SITE_NAME | Balsa Busters R/C Flying Club Field | Site name. | name |
| SITE_ID | 21607 | FAA site number. | source_id |
| CEILING / UNIT | 400 / Feet | Maximum altitude allowed at the site. | ceiling_ft (AGL) |
| BOUNDARY | 660 | Size of the site boundary in UNIT (appears to be a radius). | source_attrs |
| LATITUDE, LONGITUDE, CITY, STATE, ZIP_CODE | 41.12091, -78.96075, Brookville, PA | Location. | notes |
| LAST_EDIT_DT | 4/1/2019 | Last edit date of the record. | source_attrs |
| POC | (omitted here) | Site contact. | notes |

Rule assigned by the tool: allowed; citation 49 U.S.C. 44809(a)(5).

## 11. SEAMS – Sporting Event Automated Monitoring System

Service: SEAMS_Production_View

Layer: SEAMS (layer 0)

Source: FAA ArcGIS server services1.arcgis.com/n4Ot9Qz0t5espY4s (linked from the FAA stadium page)

Refresh group: fast (every 5 min)

Output layer: seams

Records: 447 event polygons; New York area: 39 events

What it is: Near-real-time schedule of stadium TFRs for each scheduled game or race, supplied by the leagues to the FAA, with a 3 NM polygon per event.

What was pulled: All fields and polygon geometry.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| EVENT_NAME | Indiana @ Rutgers | Event. | name |
| VENUE | SHI Stadium | Venue. | name |
| GAME_DATE | 1791068400000 (epoch ms) | FAA alias "TFR Start": when the TFR begins (already 1 hour before the event). | window_start |
| END_DATE | 1791086400000 | FAA alias "TFR End". | window_end |
| STATUS | SCHEDULED | Event status from the league feed. | notes |
| LEAGUE_NAME | NCAA | League. | notes |
| IS_ACTIVE | 0 | 1 = TFR active right now. | notes |
| TIME_REMAINING | SCHEDULED | Text countdown for active TFRs. | notes |
| latitude / longitude | 40 30 48 88N | Venue location as text. | source_attrs |
| GAME_DETAIL_ID | 6594339 | Event ID. | source_id |
| updatedAt, EditDate, Editor, GlobalID |  | Record maintenance fields. | source_attrs |

Rule assigned by the tool: prohibited inside the window; 0 to 3,000 ft AGL.

## 12. tfr.faa.gov – TFR list

Service: https://tfr.faa.gov/tfrapi/getTfrList

Layer: JSON list

Source: FAA TFR website

Refresh group: fast

Output layer: (used to build tfr_active)

Records: ≈94 active TFRs listed today; New York area: —

What it is: The list of current TFRs shown on tfr.faa.gov.

What was pulled: The whole list; the tool uses two fields.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| notam_id | 6/6672 | NOTAM number (year digit / sequence). | Joins to shapes and detail files |
| mod_abs_time | 202610011841 | Last modification time, YYYYMMDDhhmm UTC. | source_last_edit; cache key |
| type, facility, state, description, mod_date, is_new, gid | HAZARDS, ZLC, MT, … | Category, responsible ARTCC, state, summary text, etc. | Not used |

## 13. tfr.faa.gov – TFR shapes (GeoServer WFS)

Service: TFR:V_TFR_LOC

Layer: WFS feature type

Source: tfr.faa.gov/geoserver

Refresh group: fast

Output layer: tfr_active

Records: 105 shapes today; New York area: 3 shapes

What it is: The published polygon(s) for each TFR.

What was pulled: All features, all properties, polygon geometry (EPSG:4326).

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| NOTAM_KEY | 6/6666-1-FDC-F | NOTAM number plus part/series suffix (FDC = Flight Data Center NOTAM). | source_id; joins to details |
| TITLE | East Rutherford, NJ, Friday, October 2, 2026 … | Human-readable title. | name |
| LEGAL | SECURITY | TFR category (VIP, SECURITY, HAZARDS, SPACE OPERATIONS, AIR SHOWS/SPORTS, UAS PUBLIC GATHERING). | notes |
| CNS_LOCATION_ID | ZNY | Responsible ARTCC (ZNY = New York Center). | source_attrs |
| LAST_MODIFICATION_DATETIME | 202610011809 | Last change, YYYYMMDDhhmm. | cache key |
| STATE, GID | NJ, 234267 | State; internal ID. | source_attrs |

## 14. tfr.faa.gov – TFR detail XML (XNOTAM-Update)

Service: https://tfr.faa.gov/download/detail_<n>_<nnnn>.xml

Layer: One XML file per TFR

Source: tfr.faa.gov

Refresh group: fast (cached until the TFR changes)

Output layer: tfr_active

Records: 1 file per TFR; New York area: —

What it is: The full NOTAM in structured form: times, areas, altitudes and text.

What was pulled: These elements are read; everything else is ignored.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| Not/dateEffective, Not/dateExpire | 2026-10-03T12:00:00 | Start / end of the TFR. Checked against the NOTAM text: these are UTC even when codeTimeZone says e.g. EDT. | window_start / window_end |
| Not/codeTimeZone | EDT | Time-zone label. Ignored for the reason above. | Not used |
| dateIssued | 2026-10-01T15:41:36 | When the NOTAM was issued. | Fallback for window_start |
| TfrNot/codeType | 91.141 | The CFR section the TFR is issued under (91.137 disaster, 91.141 VIP, 91.143 space ops, 91.145 events, 99.7 security). | citation |
| aseTFRArea/txtName | Area A | Name of each area in the TFR. | notes |
| codeDistVerLower / Upper | HEI / ALT | Altitude reference: HEI = height above ground (AGL), ALT = altitude above sea level (MSL). | floor_ref / ceiling_ref |
| valDistVerLower / Upper, uomDistVer… | 0 / 9999, FT | Altitude values and units (FT or FL). | floor_ft / ceiling_ft |
| txtDescrTraditional | !FDC 6/6618 ZDC VA..AIRSPACE … | Full NOTAM text. | notes |

Not pulled / not used: The XML also carries the area geometry (Avx vertex lists, arcs and circles); the tool uses the WFS shapes instead. When a TFR has several areas, the shapes cannot be matched one-to-one with the areas, so the tool reports the overall lowest floor and highest ceiling and lists every area in notes.

## 15. U.S. Airports (map app only)

Service: US_Airport

Layer: Airports (layer 0)

Source: FAA ArcGIS server

Refresh group: loaded by uas_app.py, cached 7 days

Output layer: Airport icons and "nearest airport"

Records: 19,535 points; New York area: 1,219 points (258 public-use airports)

What it is: Airports, heliports, seaplane bases and other landing facilities shown on FAA charts.

What was pulled: Filter OPERSTATUS not CLOSED; fields IDENT, ICAO_ID, NAME, TYPE_CODE, PRIVATEUSE, MIL_CODE, SERVCITY, STATE, ELEVATION.

| Field (FAA name) | Example value | What it means | How the tool uses it |
| --- | --- | --- | --- |
| IDENT / ICAO_ID | JFK / KJFK | FAA and ICAO identifiers. | Icon label |
| NAME | John F Kennedy Intl | Name. | Briefing |
| TYPE_CODE | AD | AD = airport, HP = heliport, SP = seaplane base, GL = gliderport, UL = ultralight, BP = balloonport. | Blue icon for AD |
| PRIVATEUSE | 0 | 1 = private use. | Private fields go to a separate layer |
| MIL_CODE | CIVIL | CIVIL, MIL or ALL (joint). | Military icon color |
| SERVCITY, STATE, ELEVATION | NEW YORK, NY, 13 | City served, state, field elevation (ft MSL). | Kept |
| OPERSTATUS | OPERATIONAL | Operational status. | Filter |

## 16. FAA NOTAM API (set up, not yet pulled)

Service: https://external-api.faa.gov/notamapi/v1/notams

Layer: REST API (geoJson)

Source: FAA API portal (api.faa.gov)

Refresh group: fast

Output layer: notam_tfr

Records: —; New York area: — (reported as skipped: no API keys)

What it is: The FAA's NOTAM system. The tool would request FDC NOTAMs and keep TFR NOTAMs issued under 91.141 (VIP), 91.137/91.138 (disaster) and 91.143 (space operations).

What was pulled: Query: classification=FDC, responseFormat=geoJson, pageSize=1000. Fields it would read from properties.coreNOTAMData.notam: id, number, text, location, effectiveStart, effectiveEnd, minimumFL, maximumFL, coordinates, radius, lastUpdated.

Rule assigned by the tool: Requires FAA_NOTAM_CLIENT_ID / FAA_NOTAM_CLIENT_SECRET. Untested because no keys were available.

## Non-FAA services

| Service | Used for | What is pulled |
| --- | --- | --- |
| Esri ArcGIS Online basemaps (World Dark Gray, World Light Gray, World Imagery, reference labels) | Background map tiles | Map images only; no data |
| Open-Meteo forecast API | Weather chip and wind in the briefing (Streamlit app) | current: temperature_2m, wind_speed_10m, wind_gusts_10m, wind_direction_10m, weather_code |
| Leaflet 1.9.4 (unpkg CDN) | Map display library | Code only |

# 3. FAA datasets not used

All 95 other services on the same FAA ArcGIS server, grouped by why they were left out. Record counts are from 1 October 2026. Descriptions in quotes are the FAA's own service descriptions; the others are based on the field names and sample records.

## 3.1 Drone-relevant: candidates to add

| Service | Records | What it contains | Why not used |
| --- | --- | --- | --- |
| Airspace_Schedule (table) | 271 | Operating hours for part-time airspace. Fields FAA_ID, Airspace_ID, APPLIES (XML timesheet, e.g. ACK 06:00–21:00, UTC-5, DST adjusted). | Not joined yet. Could tell users when a part-time Class D tower closes and the airspace changes. |
| UAS_NSR_Pending | 12 | Pending national security UAS restrictions (same fields as #5), not yet in force. | Not current rules. Could be shown as "coming soon". |
| Pending_Part_Time_National_Security_UAS_Flight_Restrictions | 5 | Pending version of #6. | Not current. |
| Digital_Obstacle_File | 656,884 points | Obstacles (towers, rigs, buildings) with AGL and AMSL heights, lighting and marking (OAS_Number, Type_Code, AGL, AMSL, Lighting, Marking…). | A hazard, not a rule. Good candidate for a safety layer. |
| MTRSegment | 3,654 lines | Military Training Routes, many with low floors (sample: 500 ft AGL to 6,000 ft MSL, 5 NM either side). | Line features, advisory only. Candidate advisory layer. |
| Operating_Categories_Map / Operating_Category_Night_Data | 76,039 / 49,130 | Polygons with one attribute, POP_DENSITY (e.g. "CATEGORY 2", "CATEGORY 3"). FAA gives no further description; it appears to be population-density categories for drone operations (possibly tied to proposed BVLOS rules). | Meaning not documented; not a current airspace rule. |
| Boundary_Airspace | 477 | ARTCC and special air-traffic-rule areas (e.g. Anchorage terminal area). | Not drone-specific. |
| Runways, RunwayArea, RunwayLine, Buffer of Runways | 23,368 / 41,667 / 41,668 / 32 | Runway outlines and center lines; a small runway-buffer analysis layer. | Not rules; could be used for detail. |
| Airspace | 16,789 | Older combined airspace layer with a different schema (NAME_TXT, VERTLIMITS_TXT, WORKHRRMK_TXT; edit dates around 2015). | Appears superseded by Class_Airspace and Special_Use_Airspace. |
| ADHP | 36,367 | Older airport/heliport points (IDENT_TXT, NAME_TXT, AIRANALYSIS_CODE). | Appears superseded by US_Airport. |

## 3.2 Duplicates, test copies and pending (next-cycle) data

| Service(s) | Records | What it is |
| --- | --- | --- |
| FAA_UAS_FacilityMap_Data_Primary, FAA_UAS_FacilityMap_Data_Secondary | 371,352 each | "Test Dataset for USS Onboarding" (copies for LAANC providers). |
| FAA_UAS_FacilityMap_Data_Test, FAA_UAS_FacilityMap_Data_V5, FAA_UAS_FacilityMap_Data_V5_AppTest, FAA_UAS_FacilityMap_Data_V5_Dev, FAA_UAS_FacilityMap_Data_AppTest, FAA_UAS_FacilityMap_Data_UUID | 368k–378k | Test, development and versioned copies of the facility map (UUID = point version). |
| Part_Time_National_Security_UAS_Flight_Restrictions_Primary, Part_Time_National_Security_UAS_Flight_Restrictions_Secondary | 5 each | Copies of #6 for onboarding. |
| Test_Class_Airspace, Test_AMA_Site_Data | 6,061 / 1,940 | Test copies of Class Airspace and fixed sites. |
| Airspace_Schedule_Preview | 323 | "Airspace Schedule Onboarding Data". |
| Pending / next-cycle (31 services) | varies | Next-cycle (not yet effective) versions of aeronautical layers: Pending_Class_Airspace, Pending_Special_Use_Airspace, Pending_Airports, Pending_Boundary_Airspace, Pending_ATS_Route, Pending_MTR_Segment, Pending_Runway, Pending_NAVAID, Pending_Navaid_Component, Pending_Designated_Point, Pending_ILS_System, Pending_ILS_Component, Pending_Holding_Patterns, Pending_Radials_Bearing, Pending_ChangeOver, Pending_Route_Airspace, Pending_Services, Pending_Notes, Clearance_Pending, EnrouteInformationPending, FrequencyPending, RoutePortionPending, Pending_AM_Apron, Pending_AM_Beacon, Pending_AM_Building, Pending_AM_Hotspot, Pending_AM_Runway, Pending_AM_Special_Movement, Pending_AM_Stopway, Pending_AM_Taxiway, Pending_AM_Wind_Indicator. |

Why not used: these repeat datasets that are already used (or are their test and development copies), or hold data that is not yet effective.

## 3.3 Instrument-flight (IFR) aeronautical data

| Service(s) | What it is |
| --- | --- |
| ATSRoute, ATS_Route, Route_Airspace, RoutePortion (table), EnrouteInformation (table) | Airways (Victor/Jet/RNAV routes) and their segments for instrument flight. |
| DesignatedPoint, DesignatedPoints | Named navigation fixes / waypoints. |
| NAVAIDSystem, NavaidComponent, NavaidComponents | Radio navigation aids (VOR, NDB, DME) and their parts. |
| ILSSystemNew, ILS_Component | Instrument Landing Systems. |
| HoldingPattern, RadialBearing, ChangeOver | Holding patterns, published radials/bearings and changeover points on IFR charts. |
| Frequencies (table), Services, Notes | Radio frequencies, ATC services and chart notes. |
| Clearance, Clearances | "Clearance Information" (charted clearance data). |
| EnrouteStatus | Internal progress view of the FAA en-route automation process. |

Why not used: these describe the instrument-flight system for crewed aircraft (routes, fixes, radio aids) and contain no drone restrictions.

## 3.4 Airport diagrams, Canada and working layers

| Service(s) | What it is |
| --- | --- |
| AM_Apron, AM_Beacon, AM_Building, AM_Hotspot, AM_Runway, AM_Special_Movement, AM_Stopway, AM_Taxiway, AM_Wind_Indicator, AM_Taxiway_Statistics | Airport mapping (airport diagram) features: ramps, buildings, taxiways, hotspots. |
| CYAM_CYXX_ClassD_over_US | Canadian (NAV CANADA) Class D areas that extend over the U.S. (6 polygons). |
| Nav_Canada_UASFM_Publication | NAV CANADA drone facility map (2,266 cells), Canadian equivalent of the LAANC grid. |
| Matt_FL, survey123_6f46319e73084d5da889f61ae578b303_fieldworker, Quick_example_Nofly_areas, FAA5010_NFDC.gdb - NFDC_Airports visible areas | Working, survey or example layers (1,002, 257, 0 and 2 features). |

Why not used: airport-surface detail is not airspace, Canadian data falls outside U.S. rules, and the working layers are internal or empty.